Project Documentation: Grey-Flow Hydro

Sub-title: Greywater Hydroponics Monitoring & Implementation Guide

1. Executive Summary

Grey-Flow Hydro is a DIY hydroponic solution designed to recycle household greywater (from showers, sinks, and laundry) to cultivate fresh produce. By integrating a dedicated monitoring system, this project ensures that recycled water is treated and balanced effectively, making it suitable for growing leafy greens and herbs while reducing fresh water consumption by up to 90%.

2. Important Safety Warnings ⚠️

This is an experimental hobbyist project. Please adhere to the following safety guidelines:

- Source Control: Use only greywater. Never use "black-water" (toilets) or greasy kitchen water.

- Soap Selection: Use only biodegradable, plant-safe soaps (low sodium, no boron).

- Crop Limitation: Grow only leafy greens (lettuce, basil, etc.). Do not grow root vegetables.

- Hygiene: Thoroughly wash all produce before consumption.

- Non-Potable: Do not drink the system water.

- Regulation: Check local laws regarding greywater reuse in your area.

3. The Monitoring System

To ensure plant health and safety, the following parameters must be tracked before the water enters the hydroponic channels.

3.1 Key Parameters to Track

| Parameter | Target Range | Purpose |
| --- | --- | --- |
| pH Level | 5.8 – 6.5 | Ensures optimal nutrient absorption. |
| EC / TDS | 800 – 1500 ppm | Measures nutrient and salt concentration. |
| Temperature | 18°C – 24°C | Prevents root stress and disease. |
| Oxygen (DO) | High | Essential for root respiration. |

3.2 Implementation Levels

- Manual (Entry Level): Use a handheld 3-in-1 digital meter to test the reservoir daily.

- Automated (Advanced): Use an Arduino or Raspberry Pi with submerged sensors (pH, DS18B20 temp, and EC) to provide real-time data to a mobile app or LCD.

4. System Components & Materials

4.1 Filtration & Collection

- Collection tank/bucket.

- Settling barrel (20–50 gallons) with sand and gravel filtration layers.

4.2 Hydroponic Infrastructure

- NFT Channels (PVC pipes or gutters) set at a 1–2% slope.

- Net pots and growing media (clay pebbles or rockwool).

- Submersible water pump and digital timer.

4.3 Recommended Starter Plants

- Greens: Lettuce, Spinach, Kale.

- Herbs: Basil, Mint, Cilantro.

5. Operational Workflow

- Collection: Greywater is gathered and passed through a gravel/sand filter.

- Analysis: Water is tested for pH and EC levels; adjustments are made if necessary.

- Distribution: Checked water moves to the main reservoir.

- Circulation: A pump moves a thin film of water over the plant roots.

- Recycling: Water loops back to the reservoir or is diverted to mulch.

6. Maintenance & Tips

- Start Small: Test the system with fresh tap water for the first week.

- Sensor Care: Clean sensors weekly to remove soap film buildup and re calibrate monthly.

- Aeration: If roots appear brown or slimy, add air stones to increase dissolved oxygen.

- Filter Cleaning: Flush or replace filtration media regularly to prevent clogging.

7. Daily Monitoring & Maintenance Log

- Use the table below to record your daily readings. Consistent data collection helps prevent system crashes and ensures a healthy harvest.

| Date | Time | pH (5.8–6.5) | EC/TDS (ppm) | Temp (°C) | Water Level | Action Taken (Added nutrients, pH down, etc.) |
| --- | --- | --- | --- | --- | --- | --- |